# Step 1: Network Topology Design

## Overview

In this step, you will create the physical layout of the network in Cisco Packet Tracer.

## Required Devices

### Core Network Devices (Network Operating Centre - NOC)

- **1 Router** (e.g., 2911 or 2811)

- **1 Core Switch** (e.g., 2960-24TT or 3560-24PS)

- **1 DHCP/DNS/HTTP Server** (Server-PT)

- **1 Email Server** (Server-PT)

- **1 AAA Server** (Server-PT)

### Department Switches

- **4 Department Switches** (one for each engineering department)

- Electrical Engineering Switch

- Agricultural Engineering Switch

- Civil Engineering Switch

- Mechanical Engineering Switch

### Additional Switches

- **3 Switches** for:

- Old Auditorium

- New Auditorium

- Library

### End Devices

- **Multiple PCs** (at least 2-3 per department)

- **Laptops** (for wireless connectivity)

- **Wireless Access Points** (for wireless departments)

## Topology Layout Instructions

### 1. Place Core Devices (Center of workspace)

```

[Router] <---> [Core Switch] <---> [Servers]

```

**Steps:**

1. Drag a router to the center (name it: `Admin_router`)

2. Drag a switch next to the router (name it: `Core_Switch`)

3. Place 3 servers next to the switch:

- `DHCP_DNS_Server` (will host DHCP, DNS, and HTTP services)

- `Email_Server`

- `AAA_Server`

### 2. Connect Core Devices

- Connect Router's GigabitEthernet0/1 to Core Switch's FastEthernet0/1

- Connect all servers to Core Switch (FastEthernet ports)

### 3. Place Department Switches

Place 4 switches around the core, one for each department:

- `EEESW` (Electrical Engineering)

- `AGRICSW` (Agricultural Engineering)

- `CIVSW` (Civil Engineering)

- `MECHSW` (Mechanical Engineering)

### 4. Connect Department Switches to Core

- Connect each department switch to Core Switch using FastEthernet cables

- Use different ports on Core Switch for each department

### 5. Place Additional Switches

- `OldAud_SW` (Old Auditorium)

- `NewAud_SW` (New Auditorium)

- `Library_SW` (Library)

Connect these to Core Switch as well.

### 6. Add End Devices

- Add 2-3 PCs to each department switch

- Add laptops for wireless access

- Add wireless access points where needed

### 7. Connect End Devices

- Connect PCs to their respective department switches

- Connect wireless access points to switches

- Connect laptops wirelessly to access points

## Visual Layout Reference

```

[Old Auditorium SW]

|

[New Auditorium SW]

|

[EEESW] -- [Core Switch] -- [Router] -- [Servers]

|           |

[AGRICSW]  [CIVSW]

|           |

[MECHSW]  [Library SW]

```

## Device Naming Convention

Use these exact names for consistency:

- Router: `Admin_router`

- Core Switch: `Core_Switch`

- Electrical Engineering Switch: `EEESW`

- Agricultural Engineering Switch: `AGRICSW`

- Civil Engineering Switch: `CIVSW`

- Mechanical Engineering Switch: `MECHSW`

- Old Auditorium Switch: `OldAud_SW`

- New Auditorium Switch: `NewAud_SW`

- Library Switch: `Library_SW`

## Cable Types

- **Straight-through cables**: For connecting hosts to switches, switches to routers

## Notes

- All passwords in this project use "group8" as default (change for production)

- Domain name: engcomplex.com

- DNS server IP: 172.168.4.3

- DHCP server IP: 172.168.4.3