1. PROJECT OVERVIEW

1.1 Network Architecture

This project implements a hierarchical Local Area Network for the College of Engineering using a core-distribution-access model. The design employs Router-on-a-Stick architecture with VLAN segmentation for each department.

1.2 Network Scope

- Base Network: 172.168.0.0/16 (Class B)

- Subnetting: /24 masks for each department (255.255.255.0)

- Total VLANs: 8 logical networks

- Domain Name: engcomplex.com

- DNS Server: 172.168.4.3

1.3 Design Philosophy

The network follows industry best practices for enterprise LAN design, implementing security, scalability, and maintainability while meeting the specific requirements outlined in the IJES 2017 research paper.

2. NETWORK TOPOLOGY

2.1 Core Infrastructure (Network Operations Centre)

The Network Operations Centre (NOC) houses all core network equipment and servers, forming the central nervous system of the college network.

Core Devices:

- Admin_router: Cisco 2911 router providing inter-VLAN routing

- Core_Switch: Cisco 2960-24TT managing all VLANs and trunks

- DHCP/DNS/HTTP Server: Centralized network services (172.168.4.3)

- Email Server: College email services (172.168.4.4)

- AAA Server: Authentication, Authorization, and Accounting (172.168.4.5)

2.2 Departmental Infrastructure

Each academic department has its own access switch with dedicated VLAN, providing network segmentation and security isolation.

Department Switches:

| Department | Switch Name | VLAN ID | Gateway IP |
| --- | --- | --- | --- |
| Electrical Engineering | EEESW | 10 | 172.168.0.1 |
| Agricultural Engineering | AGRICSW | 20 | 172.168.1.1 |
| Civil Engineering | CIVSW | 30 | 172.168.2.1 |
| Mechanical Engineering | MECHSW | 40 | 172.168.3.1 |
| New Auditorium | NewAud_SW | 50 | 172.168.6.1 |
| Old Auditorium | OldAud_SW | 60 | 172.168.5.1 |
| Library | Library_SW | 70 | 172.168.7.1 |

2.3 Wireless Network

Wireless Access Points are deployed in each department, providing secure WPA2-PSK wireless connectivity while maintaining VLAN segmentation.

3. IP ADDRESSING SCHEME

3.1 Subnet Allocation

The 172.168.0.0/16 network was subdivided into /24 subnets, providing 256 possible subnets with 254 usable host addresses each.

3.2 VLAN-to-Subnet Mapping

| VLAN | Department | Network Address | Usable Range | Gateway |
| --- | --- | --- | --- | --- |
| 10 | Electrical Engineering | 172.168.0.0/24 | 172.168.0.5-254 | 172.168.0.1 |
| 20 | Agricultural Engineering | 172.168.1.0/24 | 172.168.1.5-254 | 172.168.1.1 |
| 30 | Civil Engineering | 172.168.2.0/24 | 172.168.2.5-254 | 172.168.2.1 |
| 40 | Mechanical Engineering | 172.168.3.0/24 | 172.168.3.5-254 | 172.168.3.1 |
| 50 | New Auditorium | 172.168.6.0/24 | 172.168.6.5-254 | 172.168.6.1 |
| 60 | Old Auditorium | 172.168.5.0/24 | 172.168.5.5-254 | 172.168.5.1 |
| 70 | Library | 172.168.7.0/24 | 172.168.7.5-254 | 172.168.7.1 |
| 80 | Network Operations Centre | 172.168.4.0/24 | 172.168.4.12-254 | 172.168.4.1 |

3.3 Address Reservation

- .1 Address: Router interface (default gateway)

- .2-.4 Addresses: Reserved for network equipment

- .5-.254 Addresses: DHCP pool for end devices

- .255 Address: Network broadcast address

4. DEVICE CONFIGURATION STRATEGY

4.1 Core Switch Configuration Approach

The core switch serves as the central VLAN repository, containing all VLAN definitions and trunk configurations. Department switches connect via trunk links carrying only their respective VLAN traffic.

Key Configuration Elements:

- Creation of all 8 VLANs with descriptive names

- Trunk configuration to router (carrying all VLANs)

- Trunk configuration to department switches (department-specific VLANs)

- Access port configuration for servers in VLAN 80

4.2 Router Configuration Approach

The router implements Router-on-a-Stick architecture with sub-interfaces for each VLAN, enabling inter-VLAN routing while maintaining logical separation.

Key Configuration Elements:

- IP routing enabled for inter-VLAN communication

- Sub-interface creation for each VLAN with 802.1Q encapsulation

- DHCP relay configuration pointing to central DHCP server

- Security configurations including SSH and password encryption

4.3 Department Switch Configuration Approach

Each department switch is configured as an access layer device with:

- Single VLAN assignment for all user ports

- Trunk uplink to core switch carrying only the department VLAN

- Default gateway pointing to router's VLAN interface

- Wireless AP support via additional trunk port

4.4 Server Configuration Approach

All network services are centralized in the NOC with dedicated servers:

- DHCP Server: 8 address pools (one per VLAN)

- DNS Server: Domain name resolution for engcomplex.com

- HTTP Server: College website hosting

- Email Server: SMTP and POP3 services

- AAA Server: Network device authentication

4.5 Wireless Configuration Approach

Wireless Access Points are configured with:

- Consistent SSID (EngComplex_Wireless) across all departments

- WPA2-PSK security with strong passphrase

- Connection to department VLAN via trunk links

- Auto-negotiation for Ethernet connections

5. NETWORK SECURITY IMPLEMENTATION

5.1 Device Security

- Password Protection: All devices use encrypted passwords

- SSH Access: Secure shell enabled for remote management

- AAA Integration: Centralized authentication via TACACS+

- Service Hardening: Unnecessary services disabled

5.2 Network Segmentation

- VLAN Isolation: Departments separated at Layer 2

- Inter-VLAN Routing: Controlled through router ACLs (if implemented)

- Wireless Security: WPA2-PSK encryption for wireless traffic

5.3 Access Control

- Console Access: Password-protected local access

- Remote Access: SSH/Telnet with authentication

- User Authentication: AAA server for centralized management

6. PHYSICAL CONNECTIVITY

6.1 Core Connections

- Router GigabitEthernet0/1 connects to Core Switch FastEthernet0/1 (Trunk)

- All servers connect to Core Switch FastEthernet0/13-15 (Access ports, VLAN 80)

6.2 Department Uplinks

Each department switch connects to the core switch via dedicated trunk links:

- EEESW Fa0/24 → Core Switch Fa0/5 (VLAN 10)

- AGRICSW Fa0/24 → Core Switch Fa0/4 (VLAN 20)

- CIVSW Fa0/24 → Core Switch Fa0/7 (VLAN 30)

- MECHSW Fa0/24 → Core Switch Fa0/8 (VLAN 40)

- NewAud_SW Fa0/24 → Core Switch Fa0/10 (VLAN 50)

- OldAud_SW Fa0/24 → Core Switch Fa0/11 (VLAN 60)

- Library_SW Fa0/24 → Core Switch Fa0/12 (VLAN 70)

6.3 Wireless AP Connections

Wireless Access Points connect to department switches:

- Each AP connects to department switch Fa0/20 via trunk link

- APs operate in respective department VLANs

6.4 Cable Specifications

- Switch-to-Switch: Copper Straight-Through cables for trunks

- Switch-to-Router: Copper Straight-Through cable for trunk

- Switch-to-Server: Copper Straight-Through cables for access ports

- Switch-to-AP: Copper Straight-Through cables for trunk links

- Switch-to-PC: Copper Straight-Through cables for access ports

7. TESTING AND VALIDATION PROCEDURES

7.1 DHCP Functionality Test

Procedure:

- Connect PC to any department switch port

- Release and renew IP address

- Verify correct IP assignment based on VLAN

Expected Results:

- Electrical Engineering: 172.168.0.x address

- Agricultural Engineering: 172.168.1.x address

- Civil Engineering: 172.168.2.x address

- Mechanical Engineering: 172.168.3.x address

7.2 Inter-VLAN Connectivity Test

Procedure:

- From PC in one VLAN, ping gateway of another VLAN

- Test connectivity to central servers

- Verify routing through router sub-interfaces

Expected Results:

- Successful ping responses between VLANs

- Access to DHCP/DNS server (172.168.4.3)

- Access to web server via domain name

7.3 DNS Resolution Test

Procedure:

- From any PC, ping engcomplex.com

- Use nslookup to verify DNS resolution

- Test web browser access to domain

Expected Results:

- Domain name resolves to 172.168.4.3

- Web page loads successfully

- Consistent resolution across all VLANs

7.4 Wireless Network Test

Procedure:

- Connect wireless device to EngComplex_Wireless SSID

- Enter WPA2-PSK passphrase

- Obtain IP address via DHCP

- Test connectivity to network resources

Expected Results:

- Successful association with correct SSID

- IP address from department VLAN pool

- Access to network services and internet

7.5 Email Service Test

Procedure:

- Configure email client with server settings

- Send test email between users

- Verify message delivery

Expected Results:

- Successful email transmission

- Proper authentication to mail server

- Message delivery within network

8. TROUBLESHOOTING METHODOLOGY

8.1 Layer 1 Troubleshooting

Issues: Physical connectivity problems, cable faults, port status
Diagnostic Commands: Interface status checks, cable verification

8.2 Layer 2 Troubleshooting

Issues: VLAN misconfiguration, trunk problems, MAC address issues
Diagnostic Commands: VLAN brief, trunk status, MAC address table

8.3 Layer 3 Troubleshooting

Issues: IP addressing problems, routing issues, DHCP failures
Diagnostic Commands: IP interface brief, routing table, DHCP relay verification

8.4 Common Problem Resolution

- DHCP Failure: Verify router helper-address and DHCP server pools

- Inter-VLAN Routing Failure: Check router sub-interfaces and IP routing

- Wireless Connectivity Issues: Verify AP configuration and switch port VLAN

- DNS Resolution Problems: Check DNS server service and records

9. NETWORK MONITORING AND MAINTENANCE

9.1 Regular Verification Tasks

- Daily: Core device availability checks

- Weekly: DHCP pool utilization monitoring

- Monthly: Security configuration audits

- Quarterly: Performance and capacity reviews

9.2 Performance Metrics

- DHCP Response Time: < 5 seconds

- Inter-VLAN Latency: < 10 milliseconds

- DNS Resolution Time: < 2 seconds

- Network Uptime: 99.9% availability

9.3 Documentation Requirements

- Current network diagrams

- IP address allocation records

- Device configuration backups

- Change management logs

- Incident response reports

10. SCALABILITY AND FUTURE ENHANCEMENTS

10.1 Current Capacity

- 251 devices per department VLAN

- 8 departmental networks with expansion capability

- Wireless coverage in all academic areas

- Centralized service infrastructure

10.2 Expansion Options

- Additional VLANs for new departments

- Increased wireless density with more APs

- Server virtualization for additional services

- Network monitoring system implementation

10.3 Technology Migration Path

- Upgrade to gigabit infrastructure

- Implementation of IPv6 addressing

- Advanced security features (802.1X, firewall integration)

- Cloud service integration

11. PROJECT SUCCESS CRITERIA

11.1 Functional Requirements

- All departments have isolated, functional networks

- Inter-department communication via router

- Centralized services accessible from all VLANs

- Secure wireless connectivity available

11.2 Performance Requirements

- Network meets response time thresholds

- Sufficient capacity for projected growth

- Reliable operation with minimal downtime

11.3 Security Requirements

- Proper segmentation between departments

- Secure access to network devices

- Encrypted wireless communications

- Authentication for administrative access

11.4 Usability Requirements

- Transparent operation for end users

- Consistent performance across departments

- Easy troubleshooting and maintenance

- Comprehensive documentation

12. CONCLUSION

This project successfully implements a scalable, secure, and manageable Local Area Network for the College of Engineering based on the design principles outlined in the IJES 2017 research paper. The network provides:

- Departmental Isolation: Through VLAN segmentation

- Centralized Management: Via NOC-based services

- Secure Access: Through authentication and encryption

- Wireless Integration: With consistent security policies

- Future Scalability: Through hierarchical design

The implementation demonstrates proper application of networking concepts including subnetting, VLANs, inter-VLAN routing, DHCP relay, and network security - providing a practical learning platform while meeting real-world network requirements.